### Operation and # maintenance instruction ### Spare parts list

High Lift Pallet Truck

HS-T0809

EHS-T0809

HS-T0809

Frame

Lever

Tilting Forks

Handle

Crank

Parking brake

Operation and mai	ntenance instruction

High Lift Pallet Truck HS-T0809

Manual high lift truck (later “truck” only) is designed for transport and material handling, loaded on transporting platforms or pallets, on flat and solid surfaces. It enables to offtake and to base pallets with material to the maximum lifting height of the truck and to tilt a load placed in a fence pallet with max. height 400 mm, i.e. with the position of the center of gravity 200 mm over forks by + 20°. It decreases physical strain of person manipulating with material.

|  | HS-T0809 |  |
| --- | --- | --- |
| Loading capacity Length Width Height Length of the forks Width of the fork Min. height of the forks above the floor Forks width Truck weight Lifting height Bearing surface of the forks Tilting angle of the forks Lift with load (per 1 manual stroke) Lift without load (per 1 manual stroke) | 800 1630 730 1230 1140 150 87 540 220 900 1140x540 20 ≥18 ≥60 | kg mm mm mm mm mm mm mm kg mm mm ° mm mm |

Technical data

Operation instruction

The truck is supplied by the producer with its hydraulic block filled with hydraulic oil conforming to the standard ON 65 6890. It is recommended, at everyday usage, to check oil level once a month. The oil level is correct when its level is directly below the filling hole (closed with a plug) with the forks lowered into the lowest position. When refilling oil, it must be filtered through a 25 m filter (alternative refills are SHELL TELLUST T 15, MOBIL DTE 11 M, CASTROL HYSPIN AWS 15).

Banishing  the air in the hydraulic oil

When the truck is stored for a longer period or when a non-expert repair has been performed, the air can get into hydraulics and this will result in an incorect truck function. The air in hydraulics is indicated by the partial free movement of the handle while pumping, or uneven run of the forks occurs when lifting. This problem can be removed first of all by checking oil level. The control lever must be switched into its transport position (position “O”) and by pumping while in this position, air will be removed from the pump piston area. Then, switch the control lever into the lifting position (position “I”), pump the forks into their maximum height above the floor and make several more strokes. Now, the hydraulic block is correctly aired and you can lower the forks to their lowest position above the floor.

Adjusting the valve of the hydraulic block

If the control lever is in lower position and no lifting of the forks occurs while pumping, the M5 nut located on the clutch must be loosened.

Clutch

If the control lever is pressed in its upper position and no lowering of the forks occurs, the M5 nut located on the clutch  must be tightened.

Nut M5

Operation manual

A worker older than 18 years of age, who is mentally and physically suitable and knows all safety regulations regarding material handling using low lift scissors trucks and their operation, may operate the truck. Truck operating activities are provided through the handle and on it positioned control lever, which is connected to hydraulic block. While manipulating (lifting, starting) with load that is heavier than 600 kg it is necessary that two people are operating the truck.

Lifting

pic. 1

The control lever must be in its lower position (position “I”, see the picture). Moving the handle up and down, hydraulic unit generates a power needed for lifting of a load.

Transporting the load  (transport position)

The control lever must be in its middle position (position “O”). While in this position, the load height is constant. The lever is in this position fixated. The forks are in their lower position cca 200 mm over the ground.

Lowering

Press the control lever to its upper position (position “2”). When in this position, the safety valve in the hydraulic unit opens and a truck frame sinks to its lowest position. The speed of the lowering depends on how much you press the control lever.

Tilting

pic. 2

Tilting of the forks by +  20° is provided mechanicaly by using a crank (see picture 2).

1 - Crank                  2 - Trapezium screw

Loading diagram  HS-T0809

Lubricating plan

| Pos. No. | Place of lubrication | No. of places HS-T0809 | Lubricant type | Lubricant quantity | Interval |
| --- | --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 | Chain wheel bearing Forks bearing Helix Front wheel bearing Front wheel bearing Rear wheel bearing | 1 4 2 2 4 4 | T – K3 T – V2 T – V2 T – V2 T – V2 T – V2 | 0,01 kg 0,005 kg 0,005 kg 0,005 kg 0,005 kg 0,005 kg | Twice a year Once a year Once a year Once a year Once a year Once a year |

Safety regulations

When handling loads, operational procedures, which protect operators from accidents and injuries must be used. These are mainly caused by incorrectly secured loads, by exceeding the max. load capacity, or by sliding of the truck on poor transport surfaces. Doorsills and similar obstructions must be leveled by means of ramps as well as bridges between neighboring floors.

It is forbidden:

- to use the truck for handling on inclined surfaces

- to perform any repairs and maintenance on a truck carrying a load

- to handle loads with a center of gravity higher than 500 mm

- to transport the loads while lifted higher than it is specified 200 mm over the ground except

technological essential transport

Note:

- technological essential transport is offtaking and placing of the load in the place of stacking.

It is necessary:

- to keep the loading diagram while manipulating with load, which is sticked on each stacker.

## Product disposal

The user of product is after the end of technical life obligatory:

- to drain the hydraulic system and to hand an oil over for recycling

- to hand over all metal parts for recycling

3.     to put all plastic parts into communal waste system

Chain

HS-T0809

| Chain 12 A-1 | L=857 (46 links) | 1 pcs |
| --- | --- | --- |

EHS-T0809

| Chain 12 A-1 | L=857 (46 links) | 1 pcs |
| --- | --- | --- |

Piece list – hydraulic block

| No. | ###### Description | Qty. | No. | Description | Qty. |
| --- | --- | --- | --- | --- | --- |
| F201 | Pump Piston Rod | 1 | F229 | Screw | 1 |
| F202 | Spring Washer | 1 | F230 | Nut | 1 |
| F203 | Spring | 1 | F231 | Spring | 1 |
| F204 | Dust Ring | 1 | F232 | Strike Pin | 1 |
| F205 | Seal | 1 | F233 | O-Ring | 2 |
| F206 | Bushing | 2 | F234 | Dust Catcher | 1 |
| F207 | Screw | 3 | F235 | Cover with Screw | 1 |
| F208 | Washer | 3 | F236 | Seal Washer | 1 |
| F209 | Spring Washer | 3 | F237 | Adjusting Bolt | 1 |
| F210 | Bracket | 1 | F238 | Spring | 1 |
| F211 | Shaft for Roller | 1 | F239 | Spring Seat | 1 |
| F212 | Screw | 1 | F240 | Steel Ball | 1 |
| F213 | Shaft | 1 | F241 | Seal Washer | 1 |
| F214 | Pressure Roller | 1 | F242 | Screw | 1 |
| F215 | Lever of Pedal | 1 | F243 | Locking Ring | 1 |
| F216 | Elastic Pin | 1 | F244 | Y-Ring | 1 |
| F217 | Screw | 1 | F245 | Support Ring | 1 |
| F218 | Seal Washer | 1 | F246 | Piston | 1 |
| F219 | Spring | 1 | F247 | O-Ring | 2 |
| F220 | Steel Ball | 1 | F248 | Lifting Piston Rod | 1 |
| F221 | Valve Spindle of Pump | 1 | F249 | Screw | 1 |
| F222 | Seat of pump Valve | 1 | F250 | Screw | 1 |
| F223 | O-Ring | 1 | F251 | Conduit | 1 |
| F224 | Washer | 1 | F252 | O-Ring | 2 |
| F225 | Body of Pump | 1 | F253 | Cover with Screw | 1 |
| F226 | Locking Ring | 1 | F254 | Dust Ring | 1 |
| F227 | Elastic Pin | 1 | F262 | Spring | 2 |
| F228 | Lever Plate | 1 |  |  |  |

Tilting forks

| Tilting forks | Note |  |
| --- | --- | --- |
|  | Pcs | 2 2 2 4 4 4 1 2 2 1 1 1 1 1 2 1 4 1 1 4 4 4 8 4 |
|  | Description | Washer Bearing Trapezium screw holder Plane washer 12 Spring washer 12 Screw M12x80 Fork arm pin Snap ring 9 Cotter 5x30 Trapezium screw M20x4 Fork arm – welded Trapezium screw stone Washer Push cover Bolt Tilting forks – welded Bolt M5x8 Tie plate Fork carrier – welded Roller Bearing Snap ring 25 Washer 10 Pole |
|  | Pos. | HS201T HS202T HS203T HS204T HS205T HS206T HS207T HS208T HS209T HS2010T HS2011T HS2012T HS2013T HS2014T HS2015T HS2016T HS2017T HS2018T HS2019T HS2020T HS2021T HS2022T HS2023T HS2024T |
|  | No. | 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 |

Frame – welded HS-T0809

| Frame – welded HS-T0809 | This frame is welded by using more parts, which are comprised into one indemountable unit. |
| --- | --- |

EHS-T0809

Description and illustration

EHS-T0809

Handle

Tilting forks

Parking brake

Operation and mai	ntenance instruction

High Lift Pallet Truck EHS-T0809

Manual high lift truck  with electric lifting (later “truck” only) is designed for the transport, handling and stacking of material which is placed on pallets. It enables to offtake and to base pallets to the maximum lifting height of the truck by using electrohydraulic aggregate. It decreases physical strain of person manipulating with material.

|  | EHS-T0809 |  |
| --- | --- | --- |
| Loading capacity Length Width Height Length of the forks Width of the fork Min. height over the floor Forks width Spacing between the forks Truck weight Lifting height Bearing surface of the forks Forks tilting angle Lifting speed without load Lifting speed with load Lowering speed with a load Clearance above floor Traction Accu. battery Electro hydraulic aggregate | 800 1690 730 1420 1140 150 87 540 240 275 900 1140x540 20 ≥0.05 ≥0.07 ≥0.045 30 2x12V/ 60Ah 12 V, 700W | kg mm mm mm mm mm mm mm mm kg mm mm ° m/sek m/sek m/sek mm |

Technical data

Constrution description

The truck consits from the outer frame, the internal frame, the working cylinder, the hydraulic aggregate, forks and the handle. The forks are lifted by chain anchored to the outer frame, which is pushed by a piston rod of the working cylinder. The opposite end of the chain is mounted to the forks. The offtaking of the piston rod is provided through the pressure of hydraulic oil created by hydraulic aggregate. The aggregate is activated by pressing of the push-button up and down. Wheels rotation angle is 360°, each of the wheel is equipped with a brake.

These parts are installed in the electric circuit:

- electric fuse for the power circuit

- electric fuse for the control circuit

- connector socket for connecting a charger

There is the throtting valve mounted within the hydraulic circuit.

| # Electrohydraulic unit |  |
| --- | --- |
| Flow Tank Engine output Intensity | 0.75ml / rev 1L 700 W 12 V 2 x 12V / 60Ah min. 1500 min. 100 lift |
| # Source |  |
| Traction battery Amount of charging cycles Lifts/1 charging cycle |  |

Instructions to the truck usage

Oil check and filling

Truck´s working cylinder is filled with hydraulic oil RENOLIN HV - 32. If the truck is used daily, it is recommended to check oil level 1x a month. There is right amount of oil, if there is minimum amount of oil in the tank, while the forks are in the highest position. When filling hydraulic block with oil, it is necessary to filter it through 25 m filter.

Battery control

It is necessary to control battery regularly with an equipment for measuring of electrolyte density. It is recommended, when electrolyte density is below 75 % of nominal value, to recharge the battery, so it will be not depriciated. Clean and use the battery according to the battery´s instruction manual.

Always charge the battery by chargers, which refer to listed specification of required battery. A charger is delivered as extra.

# Lowering speed adjusting

In hydraulic circle, on the plate of hydraulic cylinder there is a throtle valve assembled, by which  the speed of forks lowering is set. In case, that it is necessary to change the speed of lowering, it is possible to order throtting valve with regulation screw on it (see the picture “Connecting and gripping of hydraulic unit EHS”), which can be used for speed changing.

Operation manual

This truck can be operated by a worker older than 18 years of age, who is mentally and physically suitable and knows all safety regulations regarding material handling using trucks with electric lifting and operation. Operation of the stacker is assured through two buttons.

Lifting

Forks lifting is performed by using a press button where there is an “LIFT” arrow marked.This switches the  electrohydraulic engine, which causes forks lifting.

Load transport

When neither of the press buttons is pressed, the forks are in the neutral position. The load is held in any position, which is provided by a non-return valve in hydraulic circuit. Load transport is allowed when the forks are not higher then 200 mm above the floor.

Lowering

By pressing the press-button where there is an “LOWER” arrow marked, the valve is opened and the forks are being lowered to their lowest position. Their lowering speed is regulated by the throttle valve.

Loading diagram  EHS-T0809

Lubricating plan

| Pos. No. | Place of lubrication | No. of places HS-T0809 | Lubricant type | Lubricant quantity | Interval |
| --- | --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 | Chain wheel bearing Forks bearing Helix Front wheel bearing Front wheel bearing Rear wheel bearing | 1 4 2 2 4 4 | T – K3 T – V2 T – V2 T – V2 T – V2 T – V2 | 0,01 kg 0,005 kg 0,005 kg 0,005 kg 0,005 kg 0,005 kg | Twice a year Once a year Once a year Once a year Once a year Once a year |

Safety regulations

When handling loads, operational procedures, which protect operators from accidents and injuries, must be used. These are mainly caused by incorrectly secured loads, by exceeding the max. load capacity, or by sliding of the truck on poor transport surfaces. Doorsills and other similar obstructions must be leveled by means of ramps as well as bridges between neighbouring floors.

It is forbidden:

- to use the truck for handling on inclined surfaces

- to perform any repairs and maintenance on a truck carrying a load

- to handle loads with a center of gravity higher than 500 mm

- to transport the loads while lifted higher than it is specified 200 mm over the ground except

technologically essential transport

Note:

- technologically essential transport isofftaking and placing of the load in the place of stacking.

It is necessary:

- to keep the loading diagram while manipulating with load, which is sticked on each stacker.

## Product disposal

The user of product is after the end of technical life obligatory:

- to drain the hydraulic system and to hand an oil over for recycling

- to hand over all metal parts for recycling

- to put all plastic parts into communal waste system

Diagram of electric circuit

KN                    - power connector

ZB                     - back up battery

ISB           - accumulator indicator

HM                            - hydromotor

ST                             - stop button

P                              - fuse 160 A

CHM                    - hydromotor coil

MS                           - microswitch

A                           - accumulator

P1                             - fuse 6,3 A

EMHR - hydromotor electromagnet

TZ                           - lifting button

TS                      - lowering button

V1-V10                        - conductors

Connection and gripping of hydraulics for EHS

| Connection and gripping of hydraulics for EHS | Note |  |
| --- | --- | --- |
|  | Pcs | 1 1 1 2 1 2 2 2 |
|  | Description | Working cylinder – ass. Clamp Compression hose Straight hub Hydraulic aggregate option – hydraulic aggregate Screw M10x35 Nut M6 Washer 6,3 |
|  | Pos. | EHS101 EHS102 EHS103 EHS105 EHS106 EHS108 EHS109 EHS110 |
|  | NO. | 1 2 3 5 6 8 9 10 |

Working cylinder assembly for EHS

| No. | Name | Quantity |
| --- | --- | --- |
| 21 | Dust ring | 1 |
| 22 | Nut | 1 |
| 23 | Stop Collar | 1 |
| 24 | The hydraulic cylinder | 1 |
| 25 | Tubing connector | 1 |
| 26 | Tubing | 1 |
| 27 | Tubing connector | 1 |
| 28 | Gasket | 4 |
| 29 | Rod | 1 |
| 30 | Piston | 1 |
| 31 | O-Seals | 2 |
| 32 | Oriented band | 1 |
| 33 | Seals | 1 |
| 34 | Nut | 1 |

Tilting forks

| Tilting forks | Note |  |
| --- | --- | --- |
|  | Pcs | 2 2 2 4 4 4 1 2 2 1 1 1 1 1 2 1 4 1 1 4 4 4 8 4 |
|  | Description | Washer Bearing Trapezium screw holder Plane washer 12 Spring washer 12 Screw M12x80 Fork arm pin Snap ring 9 Cotter 5x30 Trapezium screw M20x4 Fork arm – welded Trapezium screw stone Washer Push cover Bolt Tilting forks – welded Bolt M5x8 Tie plate Fork carrier – welded Roller Bearing Snap ring 25 Washer 10 Pole |
|  | Pos. | HS201T HS202T HS203T HS204T HS205T HS206T HS207T HS208T HS209T HS2010T HS2011T HS2012T HS2013T HS2014T HS2015T HS2016T HS2017T HS2018T HS2019T HS2020T HS2021T HS2022T HS2023T HS2024T |
|  | No. | 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 |

Frame – welded EHS-T0809

| Frame – welded EHS-T0809 | This frame is welded by using more parts, which are comprised into one indemountable unit. |
| --- | --- |