Instruction Manual

Low Profile Lift Table

Note: Owner/Operator must read and understand this instruction

manual before using the low profile lift table.

RevB: 05/2007

Thank you very much for selecting our product. This instruction manual describes correct operating method to ensure prolonged service life. Please read and completely understand this manual before operating the low profile LIFT TABLE. Always keep this manual at an appropriate place. If the manual or warning decal is missing, please contact with dealer.

Note: This Manual has been prepared for skilled and competent personal. It provides instructions for using the product correctly and parts list. This Manual cannot replace the professional skills and expertise of the user.

1.

WARNING!  If operating the lift table improperly, a person may be seriously injured. Therefore, operate properly according to the following instruction

.

- Read & thoroughly understand the Instruction Manual completely before using. Follow all safety instructions strictly.

- It is necessary to check all safety devices before operation.

- Make sure that there are no obstacles in the working area.

- Do not put foot or hand in scissors mechanism or through frame.

- Screw the lifting eyes on the base frame before working on the lift table.

- Do not overload the lift table. Load should be distributed on the table according to relevant load distribution chart.

- Pay attention if local voltage and frequency is as same as the input specification of the lift table.

- Use the lift table on flat and solid ground.

- All the electrical connection and disconnection operations must be carried out by skilled and competent personal.

- While operation, it is forbidden to contact the moving parts of the lift table.

- While the lift table moving, it is forbidden to adjust or to move the load.

- It is forbidden to lift the load, which perhaps does harm to a person or other object.

- It is forbidden to operate the lift table while a person is under the table.

- Do not adjust the safety valve of hydraulic power pack.

- It is forbidden to operate the lift table even if there is small structure distortion.

- Do not use in an explosive or flammable place.

2.

CAUTION!  If operating the lift table improperly, a person may be injured. Therefore, operate properly according to the following instruction.

- The lift table is a movable lifter designed to lift or lower rated load. Do not use it for other purpose.

- Do not allow a person to operate the lift table, who does not understand its operation.

- It is forbidden to change the lift table without manufacturer’s written admission.

- It is necessary to use the spare parts designated by manufacturer.

- Make sure to keep a distance between the table and ambient objects enough to operate the lift table safely.

- Keep the hydraulic system under clean and safe condition.

- The hydraulic power pack features an electric lowering control. The coils must be fed with the required voltage as described on those coils. The power supply voltage should not exceed ±10% of the rated required voltage.

- Always do maintenance and routine check while the lift table is unloaded.

- The lift table is not waterproof and should be used in a dry environment.

3.daily inspection

Daily inspection is effective to find the malfunction or fault on the lift table. Before operation, check the lift table according to the following points.

CAUTION!   Do not use the lift table if any malfunction or fault is found.

- Check all the terms of WARNING and CAUTION.

- Check scratches, bending or crack on the lift table.

- Check smooth movement of the table.

- Check if there is any hydraulic oil leakage.

- Check the vertical creep of the table.

- Check if all the bolts and nuts are firmly tightened.

4.operating the lift table

  - LOADING

The maximum capacity of the lift table is 1000kg. Load should be distributed on the lift table equably.

  - Lifting the Table

CAUTION!  Do not overload the lift table.

Ensure the balance of loading. Do not load partially or concentrically.

- Screw and loose emergency stop switch.

- Push the UP button and power pack starts to work to lift the load.

- Loose the UP button and power pack stops working.

  - Lowering the Table

Warning!  Do not put foot or hand in scissors mechanism.

- Push the DOWN button and the table will lower.

- Loose the DOWN button and the table will stop.

NOTE

- The table is equipped with an aluminum guard to avoid accidental danger.

- If aluminum guard strikes an object while the table lowers, stop operation and check the lift table. After making sure no any abnormality, strike the UP button slightly and then the electric system will function as before.

- Emergency stop

- There are two methods of emergency stop as follows.

- Push down the emergency stop switch and the movement of table stops.

- Strike aluminum guard upward and the movement of table also stops.

  - Transportation

If necessary, the lift table can be transported with attached ringbolts.

- Pay attention to the maximum capacity of lifting equipment to be used.

- Keep the ringbolts with reasonableness.

5.Specifications

| Model |  |  | HU600 | HU1000 | HU1500 |
| --- | --- | --- | --- | --- | --- |
| Capacity |  | (kg) | 600 | 1000 | 1500 |
| Collapsed Height |  | (mm) | 85 | 85 | 105 |
| Max. Height |  | (mm) | 860 | 860 | 880 |
| Platform Length |  | (mm) | 1450 | 1450 | 1600 |
| Platform Width |  | (mm) | 985 | 1140 | 1180 |
| Approx. Lifting Time while Loaded Rated Capacity |  |  | ≤25 | ≤25 | ≤30 |
| Motor | Output (w) |  | 1100 | 1100 | 1500 |
|  | Voltage (V) |  | 400 |  |  |
|  | Revolution (r/min) |  | 1400 |  |  |
|  | Protection Class |  | IP.54 |  |  |
|  | Insulation Class |  | F. |  |  |
| Net Weight |  | (kg) | 207 | 280 | 380 |

6.HYDRAULIC CIRCUIT & ELECTRIC PRINCIPLE DIAGRAM

See Figure 1 & Figure 2.

Fig. 1 Hydraulic circuit

| 19 | B | TRANSFORMER | 35W 0-18-24V/200, | 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 240,380,400 |  |  |
| 18 | Q | SUBVOLTAGE OVERCURRENT GUARD LIMIT |  | 1 | USER PROVIDE FOR ONESELF |
| 17 | M | HYDRAULIC POWER PACK |  | 1 |  |
| 16 | FL | MALFUNCTION LIMIT SWITCH | ME-8104 | 7 |  |
| 15 | XL | LOWERING LIMIT SWITCH |  | 1 | USER PROVIDE FOR ONESELF |
| 14 | SL | LIFTING LIMIT SWITCH | ME-8104 | 1 |  |
| 13 | AN3 | EMERGENCY STOP |  | 1 |  |
| 12 | AN2 | DOWN BUTTON |  | 1 |  |
| 11 | AN1 | UP BUTTON |  | 1 |  |
| 10 | K3 | RELAY | 943-1C-24DS | 1 | 7A |
| 9 | K2 | RELAY | HRS2H-S-DE24V | 1 |  |
| 8 | K1 | RELAY | 943-1C-24DS | 1 | 7A |
| 7 | J1 | CONTACTOR | K6, AC24V | 1 |  |
| 6 | C | ELECTROLYTIC CAPACITOR | 470U50V | 1 | HORIZONTAL |
| 5 | V2 | SILICON DIODE | 1A | 1 |  |
| 4 | V1 | SILICON RECTIFYING BRIDGE | 5A | 1 |  |
| 3 | R3 | RESISTOR | 1.8KΩ/1W | 1 |  |
| 2 | R2 | FAST ACTING FUSE | 3A | 1 |  |
| 1 | R1 | FAST ACTING FUSE | 0.25A | 1 |  |
| NUMBER | SYMBOL | NAME | MODEL&STANDARD | QUANTITY | REMARK |

Fig. 2 Electric Principle Diagram

7.Service INSTRUCTIONS

  - Do routine check of fasteners, packing and oil leaking.

  - Do routine check of the function of the lift table.

  - Before service the lift table, make sure to turn off the AC power supply.

  - After service it is necessary to check the function of the lift table again.

  - ONLY a qualified personnel can do service work.

  - Do routine check of the micro-switches on the safety guard.

  - Do routine check of the hydraulic system by listening its noise, touch motor’s surface.

Caution: It is necessary to turn off the AC power supply before touch motor’s surface.

  - Pay attention to clear or even replace the oil filter after operating for a long time.

  - Appropriate lubrication is necessary to make the lift table work easily and have a prolonged service life.

  - Following table is recommended to service the lift table periodically.

| Content | After every 500 hours’ working or every 3 months later | After every 2000 hours’ working or every year |
| --- | --- | --- |
| Check oil level of oil tank | ☆ |  |
| Check the cleanliness of oil filter | ☆ |  |
| Fasten all the connecting parts again | ☆ |  |
| Check wear and tear of pressure oil pipes | ☆ |  |
| Check hydraulic cylinder | ☆ |  |
| Fix main parts tightly again | ☆ |  |
| Check the function of micro-switches | ☆ |  |
| Check whole working state of the lift table | ☆ |  |
| Lubricate all the joints and pivot points | ☆ |  |
| Check wear and tear of all axial bushes |  | ☆ |
| Replace hydraulic oil for the first time | Accumulated working ten hours’ |  |
| Replace hydraulic oil |  | ☆ |
| Check oil leaking |  | ☆ |
| Remark: ☆ stands for proceeding the item. |  |  |

8.TROUBLE SHOOTING

Note: Before service it is necessary to screw two eyebolts into relevant screw-holes on the basis lest the table lowers accidentally.

| ### Trouble | ##### Cause | ###### Remedy |
| --- | --- | --- |
| Table cannot lift while motor works normally | - Eyebolt has not been removed - AC voltage phrases mistake - Electromagnetic dysfunctions - The table is overloaded | - Remove eyebolt - Correct AC voltage phrase - Check the function of electromagnetic valve and repair it - Remove excessive load |
| Table cannot lift and motor does not work | - Lowering limit switch (if existed) damaged | - Replace limit switch |
| Table cannot lower | - Lowering limit switch or micro-switch on safety guard damaged - Electromagnetic valve dysfunctions - Safety guard works - Something wrong with electric circuit board | - Replace lowering limit switch or micro-switch. - Check the function of electromagnetic valve and repair it - Strike the UP button slightly - Replace electric circuit board |
| Table’s legs go over limit position (if existed) while table lowers | - Internal leaking in electromagnetic valve - Packing damaged in hydraulic cylinder | - Repair electromagnetic valve and if necessary replace it - Check and replace packing |
| Table cannot reach the highest position | - Oil not enough - Limit switch damaged | - Fill enough oil - Check and repair limit switch. If necessary, replace it |

| PART LIST HU1000 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Description | Qty | No | # Description | Qty |
| 1 | Table | 1 | 28 | Screw M5×12 | 6 |
| 2 | Safety guard | 1 set | 29 | Washer 5 | 2 |
| 3 | Nylon washer | 4 | 30 | Mounted plate | 1 |
| 4 | Bolt M6x50 | 4 | 31 | Chassis | 1 |
| 5 | Locknut M6 | 4 | 32 | Bush | 2 |
| 6 | External scissor | 1 | 33 | Anti-explosive valve | 2 |
| 7 | Bush | 2 | 34 | Cylinder tube | 2 |
| 8 | Nylon washer | 2 | 35 | Piston | 2 |
| 9 | Washer | 2 | 36 | Snap Ring 25 | 2 |
| 10 | Locknut M24 | 2 | 37 | Piston rod | 2 |
| 11 | Axial cover | 2 | 38 | Cylinder cover | 2 |
| 12 | Screw M5x16 | 4 | 39 | O-ring φ55x2.65 | 2 |
| 13 | Middle axle | 2 | 40 | Snap Ring 60 | 2 |
| 14 | Short wheel | 4 | 41 | Lager axial bush | 2 |
| 15 | Short bush | 4 | 42 | Packing Assembly | 2 set |
| 16 | Switch | 8 | 43 | O-ring φ25x2.65 | 2 |
| 17 | Washer 3 | 28 | 44 | Packing UHS25 | 2 |
| 18 | Screw M3×25 | 14 | 45 | Retaining ring 55 | 2 |
| 19 | Spring straight pin 5x50 | 4 | 46 | Roller base | 2 |
| 20 | Retaining ring for axle 20 | 2 | 47 | Spring straight pinφ6x35 | 2 |
| 21 | Long wheel | 2 | 48 | Retaining ring 22 | 4 |
| 22 | Long bush | 2 | 49 | Small axial bush | 4 |
| 23 | Nylon wheel axle Ⅰ | 2 | 50 | Roller | 4 |
| 24 | Internal scissor | 1 | 51 | Axial bush | 2 |
| 25 | Screw M6x10 | 3 | 52 | Middle roller | 2 |
| 26 | Eyebolt | 3 | 53 | Roller axle | 2 |
| 27 | Nylon wheel axle Ⅱ | 2 | 54 | Spring straight pin φ4×40 | 2 |
| 55 | Screw M3x10 | 8 | 66 | Pipe-joint | 1 |
| 56 | Spring washer 3 | 4 | 67 | Seal ring 14 | 2 |
| 57 | Washer 3 | 4 | 68 | Hydraulic power pack | 1 |
| 58 | Control switch | 1 | 69 | Washer 10 | 2 |
| 59 | Hydraulic-pump unit base | 1 | 70 | Spring washer 10 | 2 |
| 60 | Control wire | 1 set | 71 | Screw M10x20 | 2 |
| 61 | Plug | 1 set | 72 | High pressure hose Ⅰ | 1 |
| 62 | Electrical box | 1 | 73 | Plug | 1 |
| 62-1 | Screw | 6 | 74 | Socket | 1 |
| 62-2 | Cover | 1 | 75 | Switch wire | 1 set |
| 62-3 | Box | 1 | 76 | High pressure hose Ⅱ | 1 |
| 62-4 | Screw M4x10 | 13 | 77 | Nylon seal | 2 |
| 62-5 | Spring washer 4 | 13 | 78 | Ball valve | 1 set |
| 62-6 | Washer 4 | 13 | 79 | High pressure hose set | 1 |
| 62-7 | Nut M4 | 2 | 80 | O-ringφ10x1.8 | 1 |
| 62-8 | Connection block plate | 1 | 81 | Joint pin | 2 |
| 62-9 | Support | 1 | 82 | Seal ring 12 | 4 |
| 62-10 | Switch | 1 | 83 | Joint bolt | 2 |
| 62-11 | Board | 1 | 84 | Long tube assembly | 1 |
| 62-12 | Plastic joint | 5 set | 85 | T joint | 1 |
| 62-13 | Transformer | 1 | 86 | Short tube assembly | 1 |
| 62-14 | Plug for fuse | 1 | 87 | Screw M6x12 | 3 |
| 62-15 | Fuse | 1 | 88 | Spring washer 6 | 3 |
| 62-16 | Socket for fuse | 1 | 89 | Washer 6 | 3 |
| 63 | Washer 4 | 2 | 90 | Power-unit cover | 1 |
| 64 | Spring washer 4 | 2 | 91 | Plastic joint | 1 set |
| 65 | Screw | 2 | 92 | Rubber bush | 1 |

| ## PART LIST HU1500 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Description | Qty | No | Description | Qty |
| 1 | Table | 1 | 28 | Screw M5×12 | 6 |
| 2 | Safety guard | 1 set | 29 | Washer 5 | 2 |
| 3 | Nylon washer | 4 | 30 | Mounted plate | 1 |
| 4 | Bolt M6x50 | 4 | 31 | Chassis | 1 |
| 5 | Locknut M6 | 4 | 32 | Bush | 2 |
| 6 | External scissor | 1 | 33 | Anti-explosive valve | 2 |
| 7 | Bush | 2 | 34 | Cylinder tube | 2 |
| 8 | Nylon washer | 2 | 35 | Piston | 2 |
| 9 | Washer | 2 | 36 | Snap Ring 40 | 2 |
| 10 | Locknut M24 | 2 | 37 | Piston rod | 2 |
| 11 | Axial cover | 2 | 38 | Cylinder cover | 2 |
| 12 | Screw M5x16 | 4 | 39 | O-ring φ75x2.65 | 2 |
| 13 | Middle axle | 2 | 40 | Snap Ring φ80 | 2 |
| 14 | Short wheel | 4 | 41 | Lager axial bush | 2 |
| 15 | Short bush | 4 | 42 | Packing Assembly | 2 set |
| 16 | Switch | 8 | 43 | O-ring φ40x2.65 | 2 |
| 17 | Washer 3 | 28 | 44 | Packing UHS52×40×7 | 2 |
| 18 | Screw M3×25 | 14 | 45 | Retaining ring φ75 | 2 |
| 19 | Spring straight pin 5x60 | 4 | 46 | Roller base | 2 |
| 20 | Retaining ring for axle 25 | 2 | 47 | Spring straight pinφ6x40 | 2 |
| 21 | Long wheel | 2 | 48 | Screw M5×14 | 4 |
| 22 | Long bush | 2 | 49 | Spring washer 5 | 4 |
| 23 | Nylon wheel axle Ⅰ | 2 | 50 | Washer | 2 |
| 24 | Internal scissor | 1 | 51 | Small axial bush | 4 |
| 25 | Screw M6x10 | 3 | 52 | Roller | 4 |
| 26 | Eyebolt | 3 | 53 | Axial bush | 2 |
| 27 | Nylon wheel axle Ⅱ | 2 | 54 | Middle roller | 2 |
| 55 | Roller axle | 2 | 67 | Screw | 2 |
| 56 | Spring straight pin φ4×40 | 2 | 68 | Pipe-joint | 1 |
| 57 | Screw M3x10 | 8 | 69 | Seal ring 14 | 2 |
| 58 | Spring washer 3 | 4 | 70 | Hydraulic power pack | 1 |
| 59 | Washer 3 | 4 | 71 | Washer 10 | 2 |
| 60 | Control switch | 1 | 72 | Spring washer 10 | 2 |
| 61 | Hydraulic-pump unit base | 1 | 73 | Screw M10x20 | 2 |
| 62 | Control wire | 1 set | 74 | High pressure hose Ⅰ | 1 |
| 63 | Plug | 1 set | 75 | Plug | 1 |
| 64 | Electrical box | 1 | 76 | Socket | 1 |
| 64-1 | Screw | 6 | 77 | Switch wire | 1 set |
| 64-2 | Cover | 1 | 78 | High pressure hose Ⅱ | 1 |
| 64-3 | Box | 1 | 79 | Nylon seal | 2 |
| 64-4 | Screw M4x10 | 13 | 80 | Ball valve | 1 set |
| 64-5 | Spring washer 4 | 13 | 81 | High pressure hose set | 1 |
| 64-6 | Washer 4 | 13 | 82 | O-ringφ10x1.8 | 1 |
| 64-7 | Nut M4 | 2 | 83 | Joint pin | 2 |
| 64-8 | Connection block plate | 1 | 84 | Seal ring 12 | 4 |
| 64-9 | Support | 1 | 85 | Joint bolt | 2 |
| 64-10 | Switch | 1 | 86 | Long tube assembly | 1 |
| 64-11 | Board | 1 | 87 | T joint | 1 |
| 64-12 | Plastic joint | 5 set | 88 | Short tube assembly | 1 |
| 64-13 | Transformer | 1 | 89 | Screw M6x12 | 3 |
| 64-14 | Plug for fuse | 1 | 90 | Washer 6 | 3 |
| 64-15 | Fuse | 1 | 91 | Washer 6 | 3 |
| 64-16 | Socket for fuse | 1 | 92 | Power-unit cover | 1 |
| 65 | Washer 4 | 2 | 93 | Plastic joint | 1 |
| 66 | Spring washer 4 | 2 | 94 | Rubber bush | 1 |

| PART LIST HU600 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | #### Description | Qty | No | Description | Qty |
| 1 | Table | 1 | 30 | Spring straight pin 4x50 | 4 |
| 2 | Width safety guardⅠ | 1 | 31 | Axle for wheelⅠ | 2 |
| 3 | Connected-boardⅠ | 6 | 32 | Bushing | 4 |
| 4 | Long safety guardⅠ | 1 | 33 | Wheel | 4 |
| 5 | Screw M4×5 | 8 | 34 | Internal scissor | 1 |
| 6 | Connected-boardⅡ | 2 | 35 | Screw M6x10 | 3 |
| 7 | Long safety guardⅡ | 2 | 36 | Eyebolt | 3 |
| 8 | Width safety guardⅡ | 1 | 37 | Axle for wheelⅡ | 2 |
| 9 | Width safety guardⅢ | 2 | 38 | Screw M5×12 | 6 |
| 10 | Long safety guardⅢ | 1 | 39 | Washer5 | 2 |
| 11 | External scissor | 1 | 40 | Switch | 1 |
| 12 | Bushing | 2 | 41 | Mounted plate | 1 |
| 13 | Nylon washer | 2 | 42 | Chassis | 1 |
| 14 | Washer | 2 | 43 | Bush | 2 |
| 15 | Spring washer 8 | 4 | 44 | Anti-explosive valve | 2 |
| 16 | Hex bolt M8×20 | 4 | 45 | Cylinder tube | 2 |
| 17 | Axial cover | 2 | 46 | Piston | 2 |
| 18 | Screw M5x16 | 4 | 47 | Snap Ring 25 | 2 |
| 19 | Locknut M10 | 4 | 48 | Piston rod | 2 |
| 20 | Washer10 | 6 | 49 | Cylinder cover | 2 |
| 21 | Bushing | 4 | 50 | O-ring φ55x2.65 | 2 |
| 22 | Hex bolt M10×55 | 4 | 51 | Snap Ring 60 | 2 |
| 23 | Middle axle | 2 | 52 | Lager axial bush | 2 |
| 24 | Spring | 8 | 53 | Packing Assembly | 2 set |
| 25 | Buffer-board | 8 | 54 | O-ring φ25x2.65 | 2 |
| 26 | Locknut M8 | 8 | 55 | Packing UHS25 | 2 |
| 27 | Nut M4 | 14 | 56 | Retaining ring for axle 55 | 2 |
| 28 | Safety switch | 6 | 57 | Roller base | 2 |
| 29 | Fixed-board | 12 | 58 | Spring straight pinφ6x35 | 2 |
| 59 | Retaining ring for axle 22 | 4 | 73-16 | Socket for fuse | 1 |
| 60 | Small axial bush | 4 | 74 | Washer 4 | 2 |
| 61 | Roller | 4 | 75 | Spring washer 4 | 2 |
| 62 | Axial bush | 2 | 76 | Screw | 2 |
| 63 | Middle roller | 2 | 77 | Pipe-joint | 1 |
| 64 | Roller axle | 2 | 78 | Seal ring 14 | 2 |
| 65 | Spring straight pin 4×40 | 2 | 79 | Hydraulic power pack | 1 |
| 66 | Screw M3x10 | 8 | 80 | Spring washer 10 | 2 |
| 67 | Spring washer 3 | 4 | 81 | Screw M10x20 | 2 |
| 68 | Washer 3 | 4 | 82 | High pressure hose Ⅰ | 1 |
| 69 | Control switch | 1 | 83 | Plug | 1 |
| 70 | Hydraulic-pump unit base | 1 | 84 | Socket | 1 |
| 71 | Control wire | 1 set | 85 | Switch wire | 1 set |
| 72 | Plug | 1 set | 86 | High pressure hose Ⅱ | 1 |
| 73 | Electrical box | 1 | 87 | Nylon seal | 2 |
| 73-1 | Screw | 6 | 88 | Ball valve | 1 set |
| 73-2 | Cover | 1 | 89 | High pressure hose set | 1 |
| 73-3 | Box | 1 | 90 | O-ringφ10x1.8 | 1 |
| 73-4 | Screw M4x10 | 13 | 91 | Joint pin | 2 |
| 73-5 | Spring washer 4 | 13 | 92 | Seal ring 12 | 4 |
| 73-6 | Washer 4 | 13 | 93 | Joint bolt | 2 |
| 73-7 | Nut M4 | 2 | 94 | Long tube assembly | 1 |
| 73-8 | Connection block plate | 1 | 95 | T joint | 1 |
| 73-9 | Support | 1 | 96 | Short tube assembly | 1 |
| 73-10 | Switch | 1 | 97 | Screw M6x12 | 3 |
| 73-11 | Board | 1 | 98 | Spring washer 6 | 3 |
| 73-12 | Plastic joint | 5 set | 99 | Washer 6 | 3 |
| 73-13 | Transformer | 1 | 100 | Power-unit cover | 1 |
| 73-14 | Plug for fuse | 1 | 101 | Plastic joint | 1 set |
| 73-15 | Fuse | 1 | 102 | Rubber bush | 1 |