#### Operating Instructions

Parts List

# CAPACITY:1000 kgs

##### Electric high-lift Pallet Truck

Note: Operator MUST read and understand this operating instructions before use this Pallet Truck

RevB：04/2018

Specifications

| Model | DGB1CM-540 |  | DGB1CM-685 |
| --- | --- | --- | --- |
| Rated lifting capacity | 1000 kgs |  |  |
| Max. Fork height | 800 mm |  |  |
| Min. Fork height | 85 mm |  |  |
| Fork length | 1170 mm |  |  |
| Front load roller | φ75 x50mm |  |  |
| Steering wheel | φ180 x50mm |  |  |
| Fork width (outside) | 540 mm | 685mm |  |
| Lifting time with rated load | ≤25s |  |  |
| Lifting time without load | ≤15s |  |  |
| Pump times with hand | Fast: 28 Slow: 70 |  |  |
| Descending speed | ∠0.1 mm/second |  |  |
| Battery | 12V/50AH |  |  |
| Net Weight | 134 kgs | 140kgs |  |

# Safety Instructions

- Read the manual carefully before operation. Go through every procedure as required in the manual.

- Lifting the fork to its full range by 1 to 2 times before use to release the air in the hydraulic system.

- Don’t put the hand & foot under the fork at any time.

- Don’t work on the slope surface.

- The safety valve in the pump station will be automatically open when it is overloaded. Under such circumstances don’t try to lift the load any more by push the “up” button otherwise that will reduce the life time of the pump station. Any manipulation of the safety valve is strictly prohibited. All assembling, adjusting and maintenance shall only be done by industrial mechanics trained people. These people must be familiar with all safety regulations of European and national where the item is in use relating to the operation and maintenance of pedestrian controlled industrial trucks.

- Load should be put on the fork center. Side-load is strictly prohibited.

- The truck shall not be used to transport persons.

- It is recommended to the driver to wear security shoes.

- Switch off the electricity after use.

- To replace or fill hydraulic oil, the fork should be lowed to its bottom position. The oil to be used should be clean and filtered. The condensation point of the oil should be ≤-20℃, the motion viscosity 20-30 cst.

# Installation & Adjustment

- Screw(part No.329)

- To prevent oil-leaking from oil tank during transportation, Air Screw (part No.329) is replaced with Oil-sealing Screw(part No.322) in the factory. But it has to be replaced back when you put this truck in use.

- How to change: Screw out Oil-sealing Screw, then screw in Air Screw , thus keep oil tank always connecting with atmosphere.

- Handle and electrical equipment

During transportation, the handle is disassembled from the truck and packed separately. It has to be assembled back and adjusted properly and carefully before use.

- Installation:

- Take 3 pieces of M10 Socket cap screws(No.109), Spring Lock Washers(No.108) & a Hexagon Ring Spanner from spare parts bag. Select handle to match the truck with the same fitting numbers.

- Assemble the handle on the Handle Seat (No.101) with the 3 pieces of M10 Socket cap screws(No.109) & Spring Lock Washers(No.108) ,and Use the Hexagon Ring Spanner to screw them down.

- Put the Adjusting Bolt (No.125) in the slot at the front end of Pendulum Arm (No.321).

- Put the Release Lever(No.121) at the low position,and operate the handle(No.110) to lift the fork to 450-600mm height. Take away the Baffle(No.460) and Battery Cover(No.459), then put a Battery(No.461) into battery tank in the front of part(422). Connect Battery wire to the battery accordingly, and tighten the bolt on the Battery Clip. Place Baffle and Battery Cover in. Finally descend fork to its lowest position.

- Insert Connector Plug(No.131) into Connector Plug[No.434  under Motor Cover(No.438)]. Check all electric components such as wire ,plug etc. to see if there is any loose ,break and short-cut. If everything is normal,switch on the Switch(No.127), check the Circuit Module(No.455) ,the electricity should be over 3 lines, If below 3 lines(in the red light area), the electricity should be charged.

- Check pipes in hydraulic system,see if there is any leakage or seeping.

- Place Release Lever(No.121) on Position I,press Up-Button (No.117) intermittently, the fork should be able to ascend accordingly. Observe electrical parts and hydraulic system to see any abnormality. If every thing is normal, press Button(No.117) to lift the fork to the highest position. The Spacing should effect. Then place Release Lever(No.121) to its highest position III, the fork should be able to descend smoothly.

- Repeat the above said procedures for 2 or 3 times to see if there is any abnormality and to drive air from hydraulic system.

# Adjustment

- There are 3 different positions with different functions for the Release Lever

(part No.121).

See picture 1

Picture  1

Position I:  Lifting for fork.

Position II: Neutral for fork

Position III: Descending the fork

- Test different functions by putting the Release Lever in 3 different positions respectively.

If it does not function properly, adjust the adjusting Nut (No.126) by the following process.

See picture 2

| Problems | Adjusting the adjusting Nut |
| --- | --- |
| On position 3, Fork does not descend | Turn Nut upward anticlockwise (+) |
| On position 1, Fork does not lifting | Turn Nut downward clockwise (-) |
| On position 2, lifting or descending | Turn Nut downward clockwise (-) or upward anticlockwise (+) |

The machine is now ready for use.

#### When to add oil

See picture 3

Low down the fork to its lowest position. Turn loose the screw (No.322 or 329).Add hydraulic oil into the oil tank. Then turn the screw tightly. Press the Up-Button (No.117), check If the fork can be pumped up to its rated highest position, if not , repeat above procedure.  Don’t add the oil up to filling mouth once. The hydraulic fluid to be used must have a quality of ISO VG22 or equivalence. Mixing of different fluids is prohibited!

##### TROUBLE SHOOTING GUIDE

| Items | Symptom | Possible cause | Aliments |
| --- | --- | --- | --- |
| 1 | Fork can not be pumped up while operate the handle | Release Lever(No.121) is not in right position. | Adjust according the way in B above |
|  |  | Exist air in the hydraulic system. | To operate the fork up-down to its full range 1-2 times with no load by manual or electrical operation |
| 2 | Fork can’t be pumped up to rated highest position. | Oil is not enough. | Add hydraulic oil. (see picture 3) |
| 3 | Fork can not descend | Blocked by foreign object. | Take away foreign object |
|  |  | Fork or other parts are damaged | Replace damaged parts. |
|  |  | Adjusting nut(No.126) not adjusted properly | - Adjust Adjusting nut(No.126) properly. (see picture 2) |
| 4 | Fork does not ascend when press switch (No.117) | Release Lever(No.121) is not in right position. | Adjust according the way in B above. |
|  |  | Switch is broken | Replace switch |
|  |  | Fuse melt down | Replace fuse(F1 443, F2 451) |
|  |  | Hydraulic pump does not work | Check motor |
|  |  | Electromagnetic Switch(No.432) is broken | Check Electromagnetic Switch |
| 5 | Motor does not work | Motor is broken | Check or replace motor |
|  |  | Connector and(or) plug-socket are loose | Check all connectors |
|  |  | Battery is worn out | Recharge battery |
|  |  | Fuse melt down | Check fuse(F1,F2) |
| 6 | Battery capacity to low | Battery is not charged enough | Recharge |
|  |  | Battery is defective | Replace the battery |
| 7 | Oil loss on hydraulic cylinder | Sealing elements are worn | Replace sealing elements |

#### DIAGRAM HYDRAULIC SYSTEM

#### POWER DIAGRAM

Spare parts list

Breakdown Drawing of Draw-bar

### PARTS LIST OF DRAW-BAR

| NO. | Description | Qty. | NO. | Description | Qty. |
| --- | --- | --- | --- | --- | --- |
| 101 | Handle Seat | 1 | 117 | Up-Button | 1 |
| 102 | Bush | 2 | 118 | Socket of button | 1 |
| 103 | Axle with Hole | 1 | 119 | Rubber Packing | 1 |
| 104 | Spacer | 2 | 120 | Handle | 1 |
| 105 | Roller | 1 | 121 | Controlling Handle | 1 |
| 106 | Bush | 2 | 122 | Chain connector | 3 |
| 107 | Pin | 1 | 123 | Release Rod | 1 |
| 108 | Spring lock washer | 3 | 124 | Chain | 1 |
| 109 | Socket cap screw | 3 | 125 | Adjusting Bolt | 1 |
| 110 | Draw - bar | 1 | 126 | Adjusting Nut | 1 |
| 111 | Pin | 4 | 127 | Switch | 1 |
| 112 | Screw | 8 | 128 | Screw | 3 |
| 113 | Cover | 1 | 129 | Socket of switch | 1 |
| 114 | Ball | 1 | 130 | Rubber band | 1 |
| 115 | Spring | 1 | 131 | Connector plug | 1 |
| 116 | Pin | 1 | 132 | Nylon Clips | 1 |

Breakdown Drawing of Optional Draw-bar

### PARTS LIST OF OPTIONAL DRAW-BAR

| NO. | Description | Qty. | NO. | Description | Qty. |
| --- | --- | --- | --- | --- | --- |
| 101 | Handle Seat | 1 | 119 | Rubber Packing | 1 |
| 102 | Bush | 2 | 120 | Handle | 1 |
| 103 | Axle with Hole | 1 | 121 | Controlling Handle | 1 |
| 104 | Spacer | 2 | 122 | Chain connector | 3 |
| 105 | Roller | 1 | 123 | Release Rod | 1 |
| 106 | Bush | 2 | 124 | Chain | 1 |
| 107 | Pin | 1 | 125 | Adjusting Bolt | 1 |
| 108 | Spring lock washer | 3 | 126 | Adjusting Nut | 1 |
| 109 | Socket cap screw | 3 | 127B | Switch | 1 |
| 110B | Draw - bar | 1 | 128 | Screw | 3 |
| 111 | Pin | 4 | 129B | Switch Box | 1 |
| 112 | Screw | 8 | 130 | Rubber band | 2 |
| 113 | Cover | 1 | 131 | Connector plug | 1 |
| 114 | Ball | 1 | 132 | Nylon Clips | 1 |
| 115 | Spring | 1 | 133 | Screw | 2 |
| 116 | Pin | 1 | 134 | O - Ring | 1 |
| 117 | Up-Button | 1 | 135 | Socket of switch | 1 |
| 118 | Socket of button | 1 | 136 | key | 1 |

Breakdown Drawing of Frame（Ⅰ）

PARTS LIST of FRAME（Ⅰ）

| NO. | Description | Qty. | NO. | Description | Qty. |
| --- | --- | --- | --- | --- | --- |
| 201 | Fork Frame | 1 | 213 | Bushing | 2 |
| 202 | Bolt | 1 | 214 | Spring Lock Washer | 2 |
| 203 | Locknut | 1 | 215 | Nut | 2 |
| 204 | Locknut | 2 | 216 | Knighthead | 2 |
| 205 | Plain Washer | 4 | 217 | Bushing | 2 |
| 206 | Spacer | 2 | 218 | Chassis Frame | 1 |
| 207 | Plain Washer | 8 | 219 | Circlip for Shaft | 4 |
| 208 | Pin | 2 | 220 | Shaft of Roller | 2 |
| 209 | Circlip for Shaft | 12 | 221 | Bearing | 4 |
| 210 | Bushing | 2 | 222 | Roller | 2 |
| 211 | Pin | 2 | 223 | Steel Roller | 4 |
| 212 | Eccentric Spindle | 2 |  |  |  |

Breakdown Drawing of Pump

# PARTS LIST OF PUMP

| NO. | Description | Qty. | NO. | Description | Qty. |
| --- | --- | --- | --- | --- | --- |
| 301 | Pin | 1 | 325 | Spring | 1 |
| 302 | Spring Seat | 1 | 326 | Ball Seat | 1 |
| 303 | Spring | 1 | 327 | Steel Ball | 1 |
| 304 | Dust Ring | 1 | 328 | Pump Housing | 1 |
| 305 | Y-Ring | 1 | 329 | Air Screw | 1 |
| 306 | Pump Cylinder | 1 | 330 | Steel Ball 5 | 1 |
| 307 | Piston | 1 | 331 | Cone Valve | 1 |
| 308 | Steel Ball 3 | 2 | 332 | Spring | 1 |
| 309 | Spring | 2 | 333 | O-Ring | 1 |
| 310 | O-Ring | 1 | 334 | Screw | 1 |
| 311 | Spring | 1 | 335 | Plain Washer | 1 |
| 312 | Steel Ball 6 | 1 | 336 | Throttle-Slice | 1 |
| 313 | O-Ring | 1 | 337 | Valve Set | 1 |
| 314 | Valve Seat | 1 | 338 | Piston Ring | 1 |
| 315 | O-Ring | 2 | 339 | Retaining Ring | 1 |
| 316 | Cylinder | 1 | 340 | Piston Rod | 1 |
| 317 | Spring-type Straight Pin | 1 | 341 | Nylon Gasket | 1 |
| 318 | Spring | 1 | 342 | O-Ring | 1 |
| 319 | O-Ring | 1 | 343 | Y-Ring | 1 |
| 320 | Release Indicator | 1 | 344 | Check Ring | 1 |
| 321 | Pendulum Arm | 1 | 345 | Cylinder Nut | 1 |
| 322 | Oil-sealing Screw | 2 | 346 | O-Ring | 1 |
| 323 | O-Ring | 2 | 347 | Dust Ring | 1 |
| 324 | Screw | 1 | 348 | Steel Ball 19 | 1 |

Breakdown Drawing of Frame（Ⅱ）

# PARTS LIST OF FRAME（Ⅱ）

| NO. | ### Description | Qty. | NO. | Description | Qty. |
| --- | --- | --- | --- | --- | --- |
| 401 | Seal Gasket | 2 | 435 | Sheath | 2 |
| 401A | Seal Gasket | 2 | 436 | Bolt | 1 |
| 402 | Connector | 1 | 437 | Bolt | 2 |
| 403 | Bolt | 1 | 438 | Motor Cover | 1 |
| 404 | Nut | 2 | 439 | Gasket | 2 |
| 405 | Oil Pipe | 1 | 440 | Screw | 2 |
| 406 | Carve Tie-in | 1 | 441 | Nut | 2 |
| 407 | Seal Gasket | 1 | 442 | Screw | 2 |
| 408 | Spring | 1 | 443 | Fuse | 1 |
| 409 | Steel Ball | 1 | 444 | Fuse Carrier | 1 |
| 410 | Seat of Valve | 1 | 445 | Bolt | 2 |
| 411 | Seal Gasket | 1 | 446 | Rubber band | 2 |
| 412 | Bushing | 1 | 447 | Wire Clip | 1 |
| 413 | Bearing | 1 | 448 | Heat Conduction Plate | 1 |
| 414 | Bushing | 2 | 449 | Charger | 1 |
| 415 | Elastic Collar | 4 | 450 | Screw | 6 |
| 416 | Bearing | 4 | 451 | Fuse | 1 |
| 417 | Wheel | 2 | 452 | Fuse Carrier | 1 |
| 418 | Holding Site for Wheel | 1 | 453 | Screw | 6 |
| 419 | Nut | 1 | 454 | Screw | 2 |
| 420 | Screw | 1 | 455 | Circuit Module | 1 |
| 421 | Bushing | 2 | 456 | Seat of Voltage Meter | 1 |
| 422 | Thrust Plate | 1 | 457 | Screw | 4 |
| 423 | Screw | 6 | 458 | Rubber band | 2 |
| 424 | Cover | 1 | 459 | Battery Cover | 1 |
| 425 | Steel Oil Pipe | 1 | 460 | Baffle | 1 |
| 426 | Nut | 2 | 461 | Battery | 1 |
| 427 | Sheath | 2 | 462 | Socket | 2 |
| 428 | Motor Assembly | 1 | 463 | Pull Handle | 1 |
| 429 | Rubber Sheath | 4 | 464 | Fixing Plate | 1 |
| 430 | Nut | 2 | 465 | Limit Switch | 1 |
| 431 | Gasket | 2 | 466 | Cover | 1 |
| 432 | Electromagnetic Switch | 1 | 467 | Screw | 2 |
| 433 | Screw | 2 | 1900 | Pump | 1 |
| 434 | Connector Plug | 1 |  |  |  |